Purifying TEV Protease

Based on protocol from Hill Lab

Revised by Emily March 2010

Day 1

- Have freshly streaked plate of TEV in BL21 from glycerol stock on LB+Amp+Chloramphenicol (Cam).

- Inoculate 3 mL 2XYT+Amp+Cam with colony and grow 6-8 hrs at 37ºC.

- Inoculate 2 flasks of 25 mL 2XYT+Amp+Cam with 50 µL of starter culture and grow overnight at 37ºC.

- Make and autoclave four baffled flasks of 500 mL LB broth (2 L total).

Day 2

- Add Amp and Cam to each 500 mL flask and inoculate with 12 mL overnight culture.  Grow at ~20ºC (leave top of shaker open).

- Monitor A600 and induce with IPTG when it reaches 0.5.  Will take   ~6-7 hrs.

- Grow overnight at 20ºC.

Day 3

- Spin down growths in pre-weighed bottles in JA-10 rotor at 5000 rpm for 15 min.

- Weigh bottles with pellets and determine weight of pellets.

- Make buffers:

Buffer A

20 mM Sodium Phosphate (NaP) pH 8

500 mM NaCl

10% glycerol

50 mM imidazole

Buffer B

20 mM NaP pH 8

500 mM NaCl

10% glycerol

300 mM imidazole

- Resuspend each pellet in 1:5 weight/volume Buffer A (for example, resuspend 5 g pellet in 25 mL buffer).  Aliquot 25 mL into 50 mL conical tubes.  Cell suspensions may be frozen here.

- Add half a tablet of protease inhibitor to each 25 mL of cells.

- French press each 25 mL tube 5 times.

- Add 25 µL DNase (1 mg/ml stock) to each 25 mL.

- Spin down crude homogenate in JA-10 rotor at 10,000 rpm for 40 min.

- Pool supernatants and syringe filter with 0.45 µm filter.

Purify His-tagged TEV on Ni column:

- Use 10 mL Ni-NTA column.  Charge column periodically by running 100 mL (10 column volumes) of water, followed by 10 mL of 0.1 M NiSO4, then 50 mL of water.

- Equilibrate column with 50 mL Buffer A.

- Load lysate onto column.  Collect flow-through.

- Wash column with 50 mL of Buffer A twice (Wash 1 and Wash 2).

- Elute TEV with 50 mL of Buffer B.

- Wash column with 50 mL of water.

- Store column in 20% ethanol.

- Take wavelength scans and remove samples from each fraction for SDS-PAGE to verify presence of protein.

- If precipitate forms in eluted TEV fraction, spin down and retain supernatant.

- Dialyze TEV fraction overnight at RT into 20 mM NaP pH 8, 500 mM NaCl, 10% glycerol, 10 mM DTT.

Day 4

- Recover TEV sample.  Take wavelength scan to determine concentration (blank with dialysis buffer).

- Aliquot into eppendorf tubes and snap freeze in dry ice/ethanol bath.

- Store at -80ºC.