Complex Exponentials are Eigenfunctions of LTI Systems

Proposition 1.6. For input xt=eiωt with LTI system with impulse response h(t) and output y(t), then,

yt=eiωth(σ)e-iωσdσ

For input xn=eiωn and discrete LTI system then output

yn=eiωnk=-∞∞h[k]e-iωk

To prove it, simply do the continuous time:

yt=xt**ht=ht**xt=hσxt-σdσ

=h(σ)eiω(t-σ)dσ=eiωth(σ)e-iωσdσ

Example 1.7. Given the differential equation

xt=yt+αy(t)

Then ht=e-αtu(t) giving

-∞∞h(σ)e-iωσdσ=0∞e-(α+iω)σdσ

=-e-(α+iω)∞0a+iω

For α>0, then the upper limit is zero and the result becomes

0∞e-(α+iω)σdσ=1a+iω

xn=yn-αy[n-1]