Properties: Time Invariance

There is a second property which this first order differential equation enjoys, the time invariance property coming from the constant coefficient properties. To the time invariance, simply stated, an input shifted results in an output shifted.

Definition 2.1. A system with input x(t) and output y(t) is time-invariant if

x(t-t0) creates output y(t-t0), for all inputs x and shifts t0.

Let us see that this is true for our differential equation for the special complex

exponentials. Choose xt=eiωt with output yt=1a+iωeiωt, then xt-t0=eiωte-iωt0. The output becomes

e-iωt1a+iωeiωt=1a+iωeiω(t-t0)=y(t-t0)

Thus we see that a simple shift of the complex exponential, the output is

shifted in the same manner. Demonstrate to yourself that the same is true for

arbitrary periodic inputs represented via the Fourier series xt=kakeiωkt.

Properties: Commutative, Causality, BIBO Stability

Proposition 1.4. Convolution has the property that

xt**ht=ht**x(t)

xk**hk=hk**x[k]

Show this by redefining the limits of the integral.

Definition 1.4. A system is causal, if the output y(t) at time t is not a function of future inputs.

If the system is causal, then this implies h(t)=0, t < 0. Alternatively, h[n]=0, n < 0. To see this,

yt=xt**ht=ht**xt=xσht-σdσ

=fxs,s>t, if h(t) ≠ 0 for t < 0

Definition 1.5. A system is BIBO stable (bounded-input, bounded output), if a bounded input implies a bounded output.

Here is a sufficient condition:

Proposition 1.5. If h(t)dt<∞ then the system is BIBO stable.

Let x(t)<B, ∀t, then

yt=xt**ht=ht**xt=hσxt-σdσ

≤Bh(σ)dσ<C

Example 1.6. Let h(t) = u(t+3), then the system is not causal.

yt=xt**ht=xσut+3-σdσ=0txσdσ

=f(xs,s>t)

Let h(t) = u(t), then system not BIBO stable. Let input x(t)=u(t), then x(t)≤1 bounded, and

yt=xt**ut=uσut-σdσ=0tdσ=t

There does not exist a bound C for all time t.